# 5.2.4. Topological Naming

Topological Naming mechanism is based on 3 components:

- History of used modeling operation algorithm

- Registering the built result in Data Framework (i.e. loading in OCAF document necessary elements of the extracted history)

- Selection / recomputation algorithm of a ‘selected’ sub-shape of the algorithm result

In order to have finally the expected result a work of these 3 components should be synchronized in terms of respecting rules of each over.

## History of algorithm

A basis of naming mechanism is a ‘correct’ history of used modeling operation. I.e. a supporting the used modeling operation algorithm should provide ‘correct’ history. Content of ‘correct’ history depends on topological result type of modeling the algorithm. History is responsible to provide all entities expecting by Selection / Re-computation mechanism for its consistent and correct work. The table provided below presents expected types of entities depending on result type.

| Result type | Type of sub-shapes to be returned by history of algorithm | Comments |
| --- | --- | --- |
| Solid or closed shell | Faces | All faces |
| Open shell or single face | Faces and edges of opened boundaries only | All faces plus all edges of opened boundaries |
| Closed wire | Edges | All edges |
| Opened wire | Edges and ending vertexes | All edges plus ending vertexes of the wire |
| Edge | Vertexes | Two vertexes are expected |
| Compound or CompSolid | To be used consequentially the above declared rule applied to all sub-shapes of the first level | Compound/CompSolid to be explored level by level until any the mentioned above types will be met |

History should return (and should track) only elementary types of sub-shapes, i.e. only Faces, Edges and Vertexes as other types (so called aggregation types: Compounds, Shells, Wires) are calculated by Selection mechanism automatically.

There are some simple exceptions for several cases. For example if Result contains ‘seam-edge’ (conical, cylindrical, spherical surfaces …) this seam-edge should be tracked by history in addition to defined before types. All degenerated entities should be filtered and excluded from consideration.

## Loading history in Data Framework

All elements returned by used algorithm according above provided rules should be put in Data Framework (or OCAF document in other words) consequently in linear order under so called ‘Result Label’. The ‘Result Label’ is a Label (TDF_label) used to keep algorithm result (Shape from TopoDS) in NamedShape attribute. During loading sub-shapes of the result in Data Framework should be used the rules of chapter “5.2.1.regitering shapes and their evolution”.  These rules also are applicable for loading main shape, i.e. the resulting shape produced by the modeling algorithm.

## Selection / Re-computation mechanism

When Data Framework is filled with all impacted entities (as data structures of a result of a current modeling operation as data structures of previous modeling operations on which the current one depends) any sub-shape of the current result can be ‘selected’, i.e. corresponding new naming data structures supporting this functionality can be produced and kept in Data Framework.

One of user interfaces for topological naming is the TNaming_Selector class. It implements as the mentioned above functionality – ‘selection’ of the sub-shape as some additional one. I.e. it can be used for:

• Storing a selected shape on a label – ‘Selection’

• Accessing the named shape – check the kept value of the shape

• Updating this naming – recomputation of the selected before shape

Selector places a new named shape with evolution SELECTED to the given label. The selector creates a "name" of the selected shape – unique description (data structure) how to find a selected topology using as resources a) the given context shape (the main shape kept on ‘Result Label’, which contains a selected sub-shape), b) its evolution and c) naming structure.

After any modification of a context shape and updating of the corresponding naming structure, you must call the method TNaming_Selector::Solve. If the naming structure (i.e. mentioned above ‘name’) is correct, the selector automatically updates the selected sub-shape in the corresponding named shape, else it fails.